Innercode R&D Pty Ltd

ABN 57 632 701 109  |  www.innercode.com.au

UART Log Evidence — Reference Summary

Zemi7 Pty Ltd  |  ABN: 54 664 363 277  |  ZEM-01  |  FY2025-2026  |  Section 355 ITAA 1997

The following bench-captured UART logs provide contemporaneous hardware evidence for Core Activities 1 and 2. All logs are filed in the Firmware folder (Google Drive) and committed to tkhdwyer82/Zemi-rnd (commit 7e565e1). Log dates correspond to actual test sessions performed during the R&D program (1 May – 30 June 2026).

Core Activity 1 — 3D Magnetic Sensor (ESP32-S3 + Melexis MLX90393)

Production Firmware: D+V3.2-PROD  |  Path: firmware/core_activity_1_magnetic_sensor/logs/

| Log File | Date | What It Shows | Hypothesis |
| --- | --- | --- | --- |
| ca1-raw-sensor-readings.log | 15 May | MLX90393 X/Y/Z µT at rest and during gesture — millisecond timestamps | CA1 hardware bring-up confirmed |
| ca1-noise-filter-comparison.log | 16 May | Noise floor A(18.7µT) B(9.2µT) C(4.1µT) D(2.3µT) — D selected | Hypothesis B filter iteration |
| ca1-pairing-state-machine.log | 20 May | V1/V2/V3 comparison — V2 rejects <150ms dwell, V3 rejects Z<70% | State machine discrimination |
| ca1-two-device-pairing.log | 22 May | Two-device pairing at 0.5m and 0.75m — sub-second timestamps, RSSI | Hypothesis A pairing at range |
| ca1-dynamic-motion-test-detailed.log | 27 May | 200 attempts: 193 SUCCESS (96.5%), 0 FP — walking, running, jumping | Hypothesis A dynamic conditions |
| ca1-false-positive-emi-test-detailed.log | 25 May | 450 EMI injections: 0 false positives — spike-gate + Z-dominance rejection | Hypothesis B <2% FP target |
| ca1-multi-device-session-detailed.log | 28 May | 12 concurrent units: 8/8 SUCCESS, 0 FP — RSSI per device, Git tag ref | Hypothesis A multi-device |

CA1 Outcome: Hypotheses A (>=95% success) and B (<2% FP rate) both validated. Production firmware locked: D+V3.2-PROD.

Core Activity 2 — Inductive Charging (TI BQ500212A / BQ51050B + NTC Thermistor)

Production Firmware: S3+X3-PROD  |  Path: firmware/core_activity_2_inductive_charging/logs/

| Log File | Date | What It Shows | Hypothesis |
| --- | --- | --- | --- |
| ca2-thermal-management.log | 5 May | 1Hz logging 120min — S3 throttle 0% to 12%, peak 36.6°C, 72.3% efficiency | Hypothesis A thermal safety |
| ca2-charging-efficiency.log | 5 May | Input/output mW at 40mm curvature, 5mm coil — avg 72.3%, 1.02Wh | Hypothesis B M2 efficiency |
| ca2-ferrite-comparison.log | 6 May | M1 flat 76.8% / M1 curved 68.4% / M2 72.3% / M3 75.1% — bend PASS | Hypothesis B M2 and M3 |
| ca2-ble-coexistence.log | 7 May | 2hr X3 gated session — avg delta -0.4%, PDR 98.3% — within all targets | Hypothesis C BLE coexistence |
| ca2-mlx-noise-during-charge.log | 5 May | Baseline 1.8µT vs active 2.9-3.4µT — avg +1.2µT, under 5µT limit | Hypothesis C MLX noise floor |
| ca2-integrated-session.log | 5-7 May | S3+M2+X3 and S3+M3+X3 concurrent 2hr — all 6 criteria met simultaneously | All hypotheses A, B, C concurrent |

CA2 Outcome: All three hypotheses validated simultaneously. Production firmware locked: S3+X3-PROD. IP67 and AS/NZS 8124 confirmed.

GitHub: tkhdwyer82/Zemi-rnd  |  Google Drive: Firmware folder  |  Total logs: 13 (CA1: 7, CA2: 6)  |  Prepared for: David Housler, Innercode  |  June 2026